Single axis thought experiment:

- Start with an infinite steel rod.

  - Speed of wave propagation for steel: c  5,050 m/s

- Place 2 access at a known distance: L.

- Input sinusoidal energy in-between the accelerometers at: f=100Hz, = 628.3 rad/s.

  - Wavelength: = c/f  50m

Find the distance from the Left most sensor (0m) to the input energy: x0

- The phase difference between the accels is calculated from the transfer function:

Δϕω = argHL,ωH0,ω = kL-x0-0-x0

Where:

- Δϕω: Phase difference between the access at input frequency (radians)

- Arg: Extract the frequency from the transfer function

- L: Separation distance of the accelerometers (meters)

- H(0): The auto power spectra of the accelerometer at position 0 (meters)

- H(1): The auto power spectra of the accelerometer at position L (meters)

- x0: The location of the input energy (meters)

- k: wavenumber, how fast the phase changes in space (think of it as our velocity)

  - k=ωc=2πfc  (m/s)

- Case for the source between the sensors: 0<x0<L

L-x0-0-x0=L-x0-x0=L-2x0

Δϕ=k L-2x0 -->  x0=L-Δϕ/k2

Issues and gotchas and other todos:

- Speed of sound is not constant:

  - Make a speed of sound map with hammer data?

- Phase wrap

  - If it bites do a first approximation with time of flight (TOF), but so far it looks like we will be ok, 100Hz is SLOW compared to 3,000 m/s!

- Multiple DOF

  - We will triangulate from 3 sensors as pairs

- Not infinite length

  - Window the data to avoid reflections

- Impulse as the input

  - Use building modes as carrier waves